STANDARD OPERATING PROCEDURE

Initial SonicWall Firewall Setup

| Document ID | Version | Status | Effective Date | Next Review |
| --- | --- | --- | --- | --- |
| SOP-NET-001 | 1.0.0 | APPROVED | 12/02/2025 | 12/02/2026 |

| DRAFT | IN REVIEW | APPROVED | PUBLISHED | ARCHIVED | DEPRECATED |
| --- | --- | --- | --- | --- | --- |

| Status | Classification | Review Cycle |
| --- | --- | --- |
| ☐ Draft ☐ In Review ☑ Approved ☐ Retired | ☐ Public ☑ Internal ☐ Confidential | ☐ Quarterly ☑ Semi-Annual ☐ Annual |

| Process Owner | [Name / Role / Department] |
| --- | --- |
| Author | System ¦ |
| Approved By | [Name / Title] |
| Department/Team | Network Administration ¦ |

# Table of Contents

1. Purpose

2. Scope

3. Definitions & Acronyms

4. Roles & Responsibilities

5. Prerequisites & Requirements

6. Procedure

7. Verification & Quality Checks

8. Troubleshooting

9. Related Documents

10. Revision History

11. Approval

# 1. Purpose

To establish a standardized, repeatable process for the initial out-of-the-box configuration of a new SonicWall firewall, ensuring all baseline security and system settings are applied before deployment.

# 2. Scope

## 2.1 In Scope

This SOP applies to all Network Technicians responsible for deploying new SonicWall firewalls for clients. It covers the process from device registration to the final pre-installation configuration step.

## 2.2 Out of Scope

This SOP does NOT cover:

[Exclusions]

# 3. Definitions & Acronyms

| Term / Acronym | Definition |
| --- | --- |
| SOP | Standard Operating Procedure |
| WAN | Wide Area Network; the port connecting to the internet. |
| LAN | Local Area Network; the port for the internal network. |
| MySonicwall Portal | The official online portal for managing SonicWall device licenses, registration, and firmware. |
| GUI | Graphical User Interface. |
| DNS | Domain Name System. |
| IPS | Intrusion Prevention System. |
| Geo-IP | IP address filtering based on geographic location. |

# 4. Roles & Responsibilities

| Role | Name/Position | Responsibilities |
| --- | --- | --- |
| Network Technician/Engineer |  | Responsible for executing all steps outlined in this SOP, ensuring the firewall is correctly configured for on-site installation. |

# 5. Prerequisites & Requirements

## 5.1 Required Access & Permissions

A new SonicWall firewall device.

Access to the MySonicwall portal with valid credentials.

A computer with an Ethernet port.

Two Ethernet cables.

Client's network circuit information sheet (containing static IP, subnet, gateway).

## 5.2 Required Tools & Resources

[Software/hardware requirements]

## 5.3 Pre-Conditions

[State requirements]

# 6. Procedure

⏱ Estimated Time: [XX minutes/hours]

## 6.1 Initial Connection and Access

Step 1: Register Device

Log in to the MySonicwall portal and register the new firewall device if it has not been done already.

Step 2: Connect Hardware

Connect the firewall's **WAN (X1)** port to an active internet connection.

Connect your computer's Ethernet port to the firewall's **LAN (X0)** port.

Step 3: Power On

Power on the SonicWall and wait for the status lights for both WAN and LAN ports to turn solid green.

Step 4: Configure Static IP

On your computer, manually configure the IPv4 network adapter settings as follows:

**IP address:** `192.168.168.2`

**Subnet mask:** `255.255.255.0`

**Default gateway:** `192.168.168.168`

**DNS Server:** `8.8.8.8` (or any other public DNS)

Step 5: Log In to Firewall

Open a web browser in incognito/private mode and navigate to `http://192.168.168.168`.

Use the default credentials to log in:

**Username:** `admin`

**Password:** `password`

You will be prompted to change the administrator password. Set a new, secure password.

When prompted for setup type, select **Manual Setup**.

## 6.2 System Configuration and Firmware Update

Step 1: Sync Licenses

Navigate to **System > Licenses**. Log in with your MySonicwall credentials to synchronize the device registration and service licenses.

Step 2: Set Firewall Name

Go to **System > Administration**. Set a descriptive name for the firewall (e.g., `Obera-Fairhope`).

Step 3: Configure Admin Timeout

On the same page, under **Login/Multiple Administrators**, change **Log out the Admin after inactivity (minutes)** to `60`.

Step 4: Verify Management Ports

Select the **Management** tab. Ensure **HTTP Port** is `80` and **HTTPS Port** is `443`.

Step 5: Download Latest Firmware

Go to the MySonicwall portal and select the firewall.

Navigate to the **Firmware** tab and download the latest stable firmware release (`.sig` file).

Step 6: Update Firmware

Return to the firewall GUI and navigate to **System > Firmware and Backups**.

Click **Upload Firmware** and select the `.sig` file you downloaded.

**Important:** Check the box to create a backup of your current settings before proceeding.

After the upload completes, click the **Boot** button next to the newly uploaded firmware version with the "Reboot with current settings" option.

The firewall will reboot, which can take 3-5 minutes.

## 6.3 Security Services Configuration

Step 1: UDP Flood Protection

Navigate to **Firewall Settings > Flood Protection**.

Select the **UDP** tab.

Check **Enable UDP Flood Protection**.

From the **UDP Flood Attack Protection Destination List** dropdown, select **LAN Subnets**.

Step 2: AppFlow Reporting

Navigate to **AppFlow > Flow Reporting > Settings**.

Toggle on **Enable Real-Time Data Collection**.

Toggle on **Enable AppFlow to Local Collector**.

Click **Accept**. The firewall will reboot to apply this change. Log back in after it comes back online.

Step 3: Gateway Anti-Virus

Navigate to **Security > Gateway Anti-Virus**.

Check **Enable Gateway Anti-Virus**.

Check **Enable Inbound and Outbound Inspection** for all listed protocols.

Step 4: Intrusion Prevention (IPS)

Navigate to **Security > Intrusion Prevention**.

Check **Enable IPS**.

Set **High Priority Attacks** to **Prevent All**.

Set **Medium Priority Attacks** to **Prevent All**.

Set **Low Priority Attacks** to **Detect All**.

Step 5: Anti-Spyware

Navigate to **Security > Anti-Spyware**.

Check **Enable Anti-Spyware**.

Set **High Priority Attacks** to **Prevent All**.

Set **Medium Priority Attacks** to **Prevent All**.

Set **Low Priority Attacks** to **Detect All**.

Ensure all protocols are selected for inspection.

Step 6: Geo-IP Filter

Navigate to **Security > Geo-IP Filter**.

Check **Block connections to/from countries selected in Countries tab**.

Go to the **Countries** tab and select high-risk countries to block (e.g., China, Russia, North Korea, Iran).

Check **Block all unknown countries**.

From the **Geo-IP Exclusion Object** dropdown, select **Default Geo-IP and Botnet Exclusion Group**.

Step 7: Botnet Filter

Navigate to **Security > Botnet Filter**.

Check **Block connections to/from botnet command and control servers**.

## 6.4 Network DNS Configuration

## 6.5 Final WAN Interface Configuration

# 7. Verification & Quality Checks

Complete the following checklist to verify successful procedure completion:

| ✓ | Verification Item | Verified By / Date |
| --- | --- | --- |
| ☐ | Confirm the device is registered and all licenses are active in the MySonicwall portal. |  |
| ☐ | Verify the firmware has been updated to the latest stable release. |  |
| ☐ | Double-check that all security services (AV, IPS, Geo-IP, Botnet) are enabled and configured as per Section 6.3. |  |
| ☐ | Confirm the WAN interface settings are transcribed correctly from the client circuit sheet before clicking the final 'Accept'. |  |

# 8. Troubleshooting

Common issues and their resolutions:

| Issue / Symptom | Possible Cause | Resolution |
| --- | --- | --- |
| Cannot access login page at 192.168.168.168. |  | 1. Verify your computer's static IP settings match Section 6.1. <br> 2. Ensure the Ethernet cable is securely connected to the LAN (X0) port. <br> 3. Confirm the firewall's LAN light is green. |
| Licenses fail to synchronize. |  | 1. Ensure the WAN port is connected to an active internet connection. <br> 2. Verify the MySonicwall portal credentials are correct. <br> 3. Reboot the firewall and try again. |
| Lost access after WAN configuration. |  | This is expected behavior. The device must now be installed on-site to be accessed via its new WAN or LAN IP address. |

## 8.1 Escalation Path

If issues cannot be resolved using this guide:

Document the issue and steps attempted

Escalate to [Tier 2 / Team Lead / Process Owner]

Contact: [Name] - [Email/Phone] - [Hours]

# 9. Related Documents

| Doc ID | Document Title | Relationship |
| --- | --- | --- |
|  | Client Network Circuit Information Sheet | Reference |

# 10. Revision History

| Version | Date | Status | Author | Change Description |
| --- | --- | --- | --- | --- |
| 1.0 | 2025-12-02 | APPROVED | System | Initial document creation from source material. |

# 11. Versioning Guidelines

This document follows semantic versioning (Major.Minor.Patch) to track changes:

| Version Type | When to Use | Example Changes |
| --- | --- | --- |
| Major (X.0.0) | Complete rewrite, fundamental process change | New platform migration, workflow redesign |
| Minor (x.X.0) | New steps added, sections reorganized | Added verification steps, new section |
| Patch (x.x.X) | Typo fixes, clarifications, minor corrections | Fixed screenshot, updated contact info |

## 11.1 Document Status Definitions

| Status | Description |
| --- | --- |
| Draft | Work in progress. Not approved for production use. |
| In Review | Submitted for stakeholder review. Pending approval. |
| Approved | Officially approved for production use. |
| Retired | No longer active. Archived for reference only. |

# 12. Approval

By signing below, I confirm this SOP has been reviewed and approved for use.

| Process Owner | Name: Signature: Date: |
| --- | --- |
| Department Head | Name: Signature: Date: |

This document is the property of OberaConnect. Unauthorized distribution is prohibited.