STANDARD OPERATING PROCEDURE

Ubiquiti Equipment Configuration for New Customers

| Document ID | Version | Status | Effective Date | Next Review |
| --- | --- | --- | --- | --- |
| SOP-NET-005 | 1.0.0 | DRAFT | Pending | [MM/DD/YYYY] |

| DRAFT | IN REVIEW | APPROVED | PUBLISHED | ARCHIVED | DEPRECATED |
| --- | --- | --- | --- | --- | --- |

| Status | Classification | Review Cycle |
| --- | --- | --- |
| ☐ Draft ☐ In Review ☑ Approved ☐ Retired | ☐ Public ☑ Internal ☐ Confidential | ☐ Quarterly ☑ Semi-Annual ☐ Annual |

| Process Owner | [Name / Role / Department] |
| --- | --- |
| Author | System ¦ |
| Approved By | [Name / Title] |
| Department/Team | Network Administration ¦ |

# Table of Contents

1. Purpose

2. Scope

3. Definitions & Acronyms

4. Roles & Responsibilities

5. Prerequisites & Requirements

6. Procedure

7. Verification & Quality Checks

8. Troubleshooting

9. Related Documents

10. Revision History

11. Approval

# 1. Purpose

To establish a standardized, repeatable process for configuring Ubiquiti/UniFi network equipment (switches, access points) for new customer deployments, ensuring proper firmware levels and consistent network configuration.

# 2. Scope

## 2.1 In Scope

This SOP applies to all Network Technicians responsible for deploying Ubiquiti UniFi equipment for clients. It covers switch pre-adoption firmware upgrades, access point deployment, and UniFi Controller adoption.

## 2.2 Out of Scope

This SOP does NOT cover:

[Exclusions]

# 3. Definitions & Acronyms

| Term / Acronym | Definition |
| --- | --- |
| SOP | Standard Operating Procedure |
| AP | Access Point |
| USW | UniFi Switch |
| SSH | Secure Shell |
| DHCP | Dynamic Host Configuration Protocol |
| OUI | Organizationally Unique Identifier (MAC address prefix) |

# 4. Roles & Responsibilities

| Role | Name/Position | Responsibilities |
| --- | --- | --- |
| Network Technician/Engineer |  | Responsible for executing all steps outlined in this SOP, ensuring equipment is correctly configured for deployment. |

# 5. Prerequisites & Requirements

## 5.1 Required Access & Permissions

Ubiquiti UniFi switch and/or access point(s)

Access to UniFi Network Controller (cloud or self-hosted)

SSH client (PuTTY or similar)

Network access to customer subnet

Customer subnet information from CUSTOMER_SUBNET_TRACKER.csv

## 5.2 Required Tools & Resources

[Software/hardware requirements]

## 5.3 Pre-Conditions

[State requirements]

# 6. Procedure

⏱ Estimated Time: [XX minutes/hours]

## 6.1 Pre-Deployment: Identify Device IP

Step 1: From Windows CMD**, run ping sweep to find active devices:
   ```cmd
   for /L %i in (1,1,254) do @(ping -n 1 -w 30 10.55.1.%i >nul && echo 10.55.1.%i is up)
   ```
   > Adjust subnet (10.55.1) to match customer network. Only responding IPs will be displayed.

2. **Then check ARP table for MAC addresses

```cmd

arp -a | findstr "10.55.1"

```

Step 3: Identify Ubiquiti devices** by MAC prefix (OUI):
   | Prefix | Manufacturer |
   |--------|--------------|
   | 24-5a-4c | Ubiquiti |
   | 28-70-4e | Ubiquiti |
   | 78-45-58 | Ubiquiti |
   | 80-2a-a8 | Ubiquiti |
   | 74-83-c2 | Ubiquiti |
   | dc-9f-db | Ubiquiti |
   | f0-9f-c2 | Ubiquiti |

4. **Alternative

Check DHCP leases on SonicWall:

Navigate to **Network > DHCP Server > Current DHCP Leases**

Look for recent leases with Ubiquiti MAC prefixes or hostnames like `U7-Pro`, `UBNT`

## 6.2 Switch Configuration (Pre-Adoption)

Step 1: SSH into the switch

```

ssh ubnt@<switch_ip>

```

Default credentials: `ubnt` / `ubnt`

Step 2: Upgrade firmware via SSH

```

upgrade <firmware_url>

```

Step 3: Firmware URLs by model

*USW-16-POE (v7.1.26):**

```

upgrade https://dl.ui.com/unifi/firmware/USMULTUSW16POE/7.1.26.15869/US.MULT.USW_16_POE_7.1.26+15869.240926.2129.bin

```

| Model | Version | Firmware URL |

|-------|---------|--------------|

| USW-16-POE | 7.1.26 | `https://dl.ui.com/unifi/firmware/USMULTUSW16POE/7.1.26.15869/US.MULT.USW_16_POE_7.1.26+15869.240926.2129.bin` |

> See `/home/mavrick/Projects/Secondbrain/reference/ubiquiti_firmware_urls.md` for complete firmware URL reference and additional models.

## 6.3 Access Point Configuration

## 6.4 UniFi Controller Adoption

Step 1: Log into UniFi Network Controller**

2. **Navigate to Devices** - Pending devices will appear automatically

3. **Adopt device

Click on pending device

Click **Adopt**

Wait for provisioning to complete

Step 4: Configure device settings

Set device name (e.g., `SFWB_Main-SW01`, `SFWB_Rockwell-AP01`)

Assign to appropriate site

Step 5: For switches - Configure ports

Set port profiles as needed

Configure VLANs if applicable

Enable PoE on AP ports

Step 6: For APs - Configure wireless

Assign to appropriate WLAN group

Set transmit power if needed

Configure band steering preferences

## 6.5 Network Configuration Standards

# 7. Verification & Quality Checks

Complete the following checklist to verify successful procedure completion:

| ✓ | Verification Item | Verified By / Date |
| --- | --- | --- |
| ☐ | Confirm device shows as "Online" in UniFi Controller |  |
| ☐ | Verify firmware is current (Controller will show update available if not) |  |
| ☐ | Test connectivity from client devices |  |
| ☐ | For APs: Verify SSID broadcast and client connection |  |
| ☐ | For switches: Verify port connectivity and PoE delivery |  |

# 8. Troubleshooting

Common issues and their resolutions:

| Issue / Symptom | Possible Cause | Resolution |
| --- | --- | --- |
| Cannot find device IP |  | 1. Verify device has power (LEDs active). 2. Check DHCP leases on SonicWall. 3. Ensure device is on same L2 network. |
| SSH connection refused |  | 1. Device may still be booting. 2. Device may already be adopted (different credentials). 3. Try default creds: ubnt/ubnt |
| Firmware upgrade fails |  | 1. Verify URL is correct and accessible. 2. Check device has internet access. 3. Try alternate firmware version. |
| Device not appearing in Controller |  | 1. Verify L2 connectivity to Controller. 2. Check if device is on different VLAN. 3. Use UniFi mobile app for local discovery. |
| Adoption fails |  | 1. Reset device to factory defaults. 2. Verify Controller is accessible from device subnet. 3. Check for IP conflicts. |

## 8.1 Escalation Path

If issues cannot be resolved using this guide:

Document the issue and steps attempted

Escalate to [Tier 2 / Team Lead / Process Owner]

Contact: [Name] - [Email/Phone] - [Hours]

# 9. Related Documents

| Doc ID | Document Title | Relationship |
| --- | --- | --- |
|  | Ubiquiti Firmware Reference: `/reference/ubiquiti_firmware_urls.md` | Reference |
|  | Network Defaults: `/reference/network_defaults.md` | Reference |
|  | Customer Subnet Tracker: `/network-config-builder/CUSTOMER_SUBNET_TRACKER.csv` | Reference |
|  | SOP-NET-001: Initial SonicWall Firewall Setup | Reference |

# 10. Revision History

| Version | Date | Status | Author | Change Description |
| --- | --- | --- | --- | --- |
| 1.0 | 2025-12-04 | APPROVED | System | Initial document creation |

# 11. Versioning Guidelines

This document follows semantic versioning (Major.Minor.Patch) to track changes:

| Version Type | When to Use | Example Changes |
| --- | --- | --- |
| Major (X.0.0) | Complete rewrite, fundamental process change | New platform migration, workflow redesign |
| Minor (x.X.0) | New steps added, sections reorganized | Added verification steps, new section |
| Patch (x.x.X) | Typo fixes, clarifications, minor corrections | Fixed screenshot, updated contact info |

## 11.1 Document Status Definitions

| Status | Description |
| --- | --- |
| Draft | Work in progress. Not approved for production use. |
| In Review | Submitted for stakeholder review. Pending approval. |
| Approved | Officially approved for production use. |
| Retired | No longer active. Archived for reference only. |

# 12. Approval

By signing below, I confirm this SOP has been reviewed and approved for use.

| Process Owner | Name: Signature: Date: |
| --- | --- |
| Department Head | Name: Signature: Date: |

This document is the property of OberaConnect. Unauthorized distribution is prohibited.