# SOP-NET-003: Cisco Device VLAN Configuration

### 1.0 Document Control

| ID | SOP-NET-003 |
| --- | --- |
| Version | 1.1 |
| Status | Final |
| Author | OberaConnect IT |
| Date | 2025-12-02 |
| Approved By |  |

### 2.0 Purpose

To define the standard procedure for configuring, verifying, and managing Virtual Local Area Networks (VLANs) on Cisco switches to ensure network segmentation and security.

### 3.0 Scope

This SOP applies to all network administration personnel responsible for configuring and managing Cisco network infrastructure within the organization.

### 4.0 Prerequisites

- Access: Administrative access to the target Cisco switch via console cable or SSH.

- Credentials: Valid username and password with privileges to make configuration changes.

- Network Knowledge: Understanding of the network design, including required VLAN IDs, names, and port assignments.

### 5.0 Procedure

#### 5.1 Access the Switch

- Connect to the switch using your preferred method (console or SSH). bash     ssh admin@<switch-ip-address>

- Enter your credentials when prompted.

#### 5.2 Enter Global Configuration Mode

- Enter privileged EXEC mode. enable

- Enter global configuration mode. configure terminal

#### 5.2.1 Configure VTP Mode (CRITICAL)

IMPORTANT: Always set VTP mode to prevent unintended VLAN propagation across the network.

- Set VTP mode to transparent (recommended for most deployments):

- vtp mode transparent
    ```
Or disable VTP entirely (Cisco IOS 15.x+):

- vtp mode off
```

VTP Mode Options: | Mode | Description | Use Case | |——|————-|———-| | transparent | Does not participate in VTP but passes VTP messages | Recommended default | | off | Completely disables VTP | Highest security | | server | Can create/modify VLANs, propagates to clients | Only in controlled environments | | client | Receives VLAN info from servers | Legacy deployments |

WARNING: Using server or client mode in an uncontrolled environment can cause VLAN database overwrites if a switch with a higher revision number is connected.

#### 5.3 Create and Name VLANs

- Create a new VLAN and enter the VLAN configuration sub-mode.

- vlan <VLAN_ID>
    ```
*Example:*

- vlan 10
```

- Assign a descriptive name to the VLAN.

- name <VLAN_Name>
    ```
*Example:*

- name VLAN_SALES
```

- Exit the VLAN configuration sub-mode. exit

- Repeat steps 5.3.1 - 5.3.3 for each required VLAN. Example: vlan 20     name VLAN_HR     exit

#### 5.4 Assign Access Ports to VLANs

- Select the interface or range of interfaces to configure.

- interface range <interface_type> <port_range>
    ```
*Example:*

- interface range fastEthernet 0/1 - 12
```

- Set the port mode to access. switchport mode access

- Assign the port to the desired VLAN.

- switchport access vlan <VLAN_ID>
    ```
*Example:*

- switchport access vlan 10
```

- Enable Spanning-Tree PortFast (allows immediate port activation for end devices): spanning-tree portfast

- Enable BPDU Guard (protects against rogue switches): spanning-tree bpduguard enable

- Exit the interface configuration sub-mode. exit

Complete Access Port Configuration Example:

interface range gigabitEthernet 0/1 - 24
 switchport mode access
 switchport access vlan 10
 spanning-tree portfast
 spanning-tree bpduguard enable
 exit

NOTE: PortFast and BPDU Guard should ONLY be enabled on access ports connected to end devices (PCs, printers, phones). Never enable on ports connecting to other switches.

#### 5.5 Configure Trunk Ports (If Required)

This step is necessary when connecting to another switch or a device that needs to handle traffic from multiple VLANs.

- Select the interface to configure as a trunk.

- interface <interface_type> <port>
    ```
*Example:*

- interface gigabitEthernet 0/1
```

- Set the port mode to trunk. switchport mode trunk

- Disable DTP negotiation (security best practice): switchport nonegotiate

- Change native VLAN from default (security - prevents VLAN hopping attacks):

- switchport trunk native vlan <unused_vlan_id>
    ```
*Example (use an unused VLAN like 999):*

- switchport trunk native vlan 999
```

- Specify which VLANs are allowed on the trunk (always limit to required VLANs):

- switchport trunk allowed vlan <vlan_list>
    ```
*Example:*

- switchport trunk allowed vlan 10,20,30
```

- Exit the interface configuration sub-mode. exit

Complete Trunk Port Configuration Example:

interface gigabitEthernet 0/48
 switchport mode trunk
 switchport nonegotiate
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30
 exit

SECURITY NOTE: - nonegotiate prevents DTP attacks where an attacker could negotiate a trunk - Changing native VLAN from 1 prevents VLAN hopping attacks - Always limit allowed VLANs to only those required

#### 5.6 Configure Management VLAN and IP Address

Set up switch management access via a dedicated VLAN.

- Create a management VLAN (if not already created): vlan 99     name MGMT     exit

- Configure the Switch Virtual Interface (SVI) for management: interface vlan 99     ip address 10.0.99.10 255.255.255.0     no shutdown     exit

- Set the default gateway: ip default-gateway 10.0.99.1

- Restrict management access to specific VLANs (optional but recommended):

- line vty 0 15
access-class MGMT-ACCESS in
exit

ip access-list standard MGMT-ACCESS
permit 10.0.99.0 0.0.0.255
deny any
exit

NOTE: Using a dedicated management VLAN (not VLAN 1) is a security best practice. Management traffic should be isolated from user traffic.

### 6.0 Verification

Use the following commands from privileged EXEC mode to verify the configuration.

- Display VLAN Information:

  - Check that VLANs are created and ports are assigned correctly.

- show vlan brief

- Display Trunk Port Information:

  - Verify that trunk ports are active and allowing the correct VLANs.

- show interfaces trunk

### 7.0 Save Configuration

To ensure the configuration persists after a device reboot, save the running configuration to the startup configuration.

- From privileged EXEC mode, execute one of the following commands:

- write memory
    ```
*or*

- copy running-config startup-config
```

- Confirm the save when prompted.

### 8.0 Contingency/Rollback

- To remove a VLAN:

- configure terminal
no vlan <VLAN_ID>
exit

- To reset a port to its default state:

- configure terminal
default interface <interface_type> <port>
exit

- To revert all changes: If changes have not been saved, reboot the switch without saving the configuration.

### 8.1 Troubleshooting

| Issue | Cause | Resolution |
| --- | --- | --- |
| VLAN not showing in show vlan brief | VLAN not created or VTP client mode | Create VLAN manually. Check VTP mode (show vtp status). Set to transparent if needed. |
| Port not in correct VLAN | Access VLAN not set or port in trunk mode | Verify with show interface switchport. Set switchport mode access and switchport access vlan X. |
| Trunk not passing VLANs | VLANs not allowed on trunk | Check show interface trunk. Add VLANs with switchport trunk allowed vlan add X. |
| Devices can’t communicate across VLANs | No inter-VLAN routing | Requires Layer 3 switch with SVIs or external router. Configure routing between VLANs. |
| STP blocking port unexpectedly | Loop detected or BPDU received on portfast port | Check show spanning-tree. If BPDU guard triggered, remove unauthorized switch and shut/no shut port. |
| Native VLAN mismatch | Different native VLANs on trunk ends | Verify native VLAN matches on both ends (show interface trunk). Set same native VLAN. |
| VTP not syncing VLANs | VTP domain mismatch or password | Check show vtp status. Ensure domain name and password match. Consider transparent mode. |
| Cannot access switch management | Wrong VLAN or no IP on SVI | Verify management VLAN SVI has IP (show ip interface brief). Check default gateway. |
| Port err-disabled | Security violation, BPDU guard, or port security | Check show interface status err-disabled. Fix cause, then shut / no shut. |
| Slow network performance | Spanning-tree convergence or duplex mismatch | Check show spanning-tree for blocking ports. Verify duplex/speed with show interface status. |

Diagnostic Commands:

! View all VLANs
show vlan brief

! Check port VLAN assignment
show interface <interface> switchport

! View trunk status
show interface trunk

! Check spanning-tree
show spanning-tree vlan <vlan_id>

! View VTP status
show vtp status

! Check interface errors
show interface <interface> | include errors|CRC|collision

! View MAC address table
show mac address-table vlan <vlan_id>

### 9.0 Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2025-12-02 | OberaConnect IT | Initial document creation from source. |
| 1.1 | 2025-12-29 | Jeremy Smith | SME Review: Added VTP mode configuration (5.2.1). Added Spanning-Tree PortFast and BPDU Guard for access ports. Added trunk security (nonegotiate, native VLAN change). Added Management VLAN section (5.6). Added Section 8.1 (Troubleshooting) with common issues and diagnostic commands. |