# Standard Operating Procedure: SOP-NET-007

Title: SonicWall Configuration Restore Version: 1.0 Author: Jeremy Smith Date: 2025-12-29

## 1.0 Purpose

To establish a standardized procedure for restoring a SonicWall firewall configuration from a backup (.exp) file. This procedure is used for disaster recovery, device replacement, or rollback after failed changes.

## 2.0 Scope

This procedure applies to all Network Technicians and Engineers responsible for: - Disaster recovery of SonicWall firewalls - Restoring configuration after firmware issues - Rolling back failed configuration changes - Device replacement (same model only)

## 3.0 Prerequisites

- Backup File: Valid .exp configuration backup file (see SOP-NET-006)

- Access: Administrative credentials to the SonicWall device

- Compatibility:

  - Backup must be from the same device model (e.g., TZ 270 → TZ 270)

  - Target firmware should be same or newer than backup source

- Network: Active connection to the SonicWall management interface

## 4.0 When to Use This Procedure

| Scenario | Use Restore? | Notes |
| --- | --- | --- |
| Device bricked after firmware upgrade | Yes | Restore after firmware rollback |
| Configuration change caused outage | Yes | Restore pre-change backup |
| Device RMA/replacement (same model) | Yes | Restore to new device |
| Moving config to different model | No | Manual reconfiguration required |
| Factory reset performed | Yes | Restore from backup |

## 5.0 Procedure

### 5.1 Pre-Restore Checklist

- Verify Backup File:

  - Confirm you have the correct .exp file

  - Check file size is reasonable (50KB - 500KB typically)

  - Verify file is not corrupted (open in text editor, should show XML)

- Document Current State:

  - Note current firmware version

  - Note current WAN IP (if accessible)

  - Export current config as additional backup (if device is accessible)

- Verify Compatibility:

  - Confirm backup is from same device model

  - Confirm target firmware is same or newer than backup source firmware

### 5.2 Restore Configuration

- Access SonicWall Management Interface:

  - Navigate to the SonicWall management IP

  - Log in with administrative credentials

  - If device is factory reset, use default IP 192.168.168.168

- Navigate to Import Settings:

  - Go to Device > Settings > Firmware & Backups

  - Locate the Import/Export Configuration section

- Import Configuration:

  - Click Import Settings (or Import Configuration)

  - Browse to and select your .exp backup file

  - Click Import

- Review Import Summary:

  - Review the import summary showing what will be restored

  - Confirm settings look correct

  - Click OK or Confirm to proceed

- Wait for Reboot:

  - The firewall will automatically reboot to apply configuration

  - This typically takes 3-5 minutes

  - Do NOT power off during this process

### 5.3 Post-Restore Verification

- Verify Access:

  - Log back into the management interface

  - Note: Management IP may have changed to restored configuration

- Verify Critical Settings:

  - ☐ WAN interface configured correctly

  - ☐ LAN interface and DHCP working

  - ☐ Internet connectivity functional

  - ☐ Firewall rules restored

  - ☐ VPN configurations (if applicable)

- Re-authenticate Services:

  - Navigate to Device > Settings > MySonicWall

  - Re-enter MySonicWall credentials if prompted

  - Verify license synchronization

- Test Connectivity:

  - Test internet access from LAN

  - Test any VPN connections

  - Verify critical applications accessible

## 6.0 Restore to Factory Reset Device

If restoring to a device that has been factory reset:

- Connect to Default IP:

  - Set your computer to static IP: 192.168.168.2

  - Connect to firewall LAN port

  - Navigate to http://192.168.168.168

- Complete Initial Setup:

  - Log in with default credentials (admin / password)

  - Change admin password when prompted

  - Select Manual Setup

- Register Device (if needed):

  - Complete device registration per SOP-NET-001 Section 6.2

  - Registration must complete before config can be fully applied

- Import Configuration:

  - Follow Section 5.2 above

  - Note: Some settings may require re-entry (MySonicWall credentials, certificates)

## 7.0 Restore to Replacement Device (RMA)

When replacing a failed device with new hardware (same model):

- Register New Device:

  - Register new device in MySonicWall portal

  - Transfer licenses from old device if needed (contact SonicWall support)

- Update Firmware:

  - Upgrade new device to match firmware version of backup

  - Or upgrade to latest stable release

- Import Configuration:

  - Follow Section 5.2 above

- Update WAN Settings:

  - If WAN IP is static, verify ISP settings are correct

  - May need to update MAC address with ISP

- Re-establish VPNs:

  - Site-to-site VPNs may need to be re-established

  - Notify remote sites of device replacement

## 8.0 Troubleshooting

| Issue | Resolution |
| --- | --- |
| Import fails with error | 1. Verify .exp file not corrupted 2. Check firmware compatibility 3. Try different browser |
| Cannot access device after restore | 1. Check if management IP changed 2. Connect to LAN and try default gateway IP 3. Factory reset and retry |
| Licenses not showing after restore | 1. Re-enter MySonicWall credentials 2. Click Synchronize in Licenses section 3. Reboot device |
| VPN not connecting after restore | 1. Verify WAN IP correct 2. Re-exchange keys with remote site 3. Check firewall rules restored |
| Services not enabled after restore | 1. Sync licenses with MySonicWall 2. Manually enable services 3. Verify subscription active |

## 9.0 Important Warnings

WARNING: Restoring a configuration will overwrite all current settings. Always export current configuration before restoring.

WARNING: Configurations are device-model specific. A TZ 270 backup cannot be restored to a TZ 370 or other model.

WARNING: If restoring to older firmware than the backup source, some settings may not apply correctly. Always restore to same or newer firmware.

## 10.0 Related Documents

- SOP-NET-001: Initial SonicWall Firewall Setup

- SOP-NET-004: Register Device in MySonicwall

- SOP-NET-006: SonicWall Configuration Backup

End of Document